AND - Logical And

Usage:  AND     dest,src

Performs a logical AND of the two operands replacing the destination with the result.

  - reg,reg

  - mem,reg

  - reg,mem

  - reg,immed

  - accum,immed

OR - Inclusive Logical OR

Usage:  OR      dest,src

Logical inclusive OR of the two operands returning the result in the destination.  Any bit set in either operand will be set in the destination.

Operands

- reg,reg

- mem,reg

- reg,mem

- accum,immed

- reg,immed

XOR - Exclusive OR

Usage:  XOR     dest,src

Performs a bitwise exclusive OR of the operands and returns

the result in the destination.

Operands

reg,reg

mem,reg

reg,mem

reg,immed

accum,immed

NOT - One's Compliment Negation (Logical NOT)

Usage:  NOT     dest

Inverts the bits of the "dest" operand forming the 1s complement.

Operands

  - reg

  - mem

CMP - Compare

Usage:  CMP     dest,src

Subtracts source from destination and updates the flags but does not save result.  Flags can subsequently be checked for conditions.

  - reg,reg

  - mem,reg

  - reg,mem

  - reg,immed

  - accum,immed

| CMP results | ZF | CF |
| --- | --- | --- |
| Destination < Source | 0 | 1 |
| Destination > Source | 0 | 0 |
| Destination = Source | 1 | 0 |

The following table assumed that the two operands being compared are signed.

| CMP results |  |
| --- | --- |
| Destination < Source | SF≠OF |
| Destination > Source | SF = OF |
| Destination = Source | ZF=1 |

Setting and Clearing Individual CPU Flags

Zero Flag

And al, 0; set the Zero flag

Or al, 1     ; clear Zero flag

Sign Flag

Or al, 80h ; set sign flag

And al, 7Fh ; clear sign flag

Carry Flag

STC - Sets the Carry Flag to 1.

CLC – Clears the Carry Flag.

CMC - Complement (inverts) the Carry Flag .

Overflow Flag

Mov al, 7Fh

Add al,1   ; al = 80h (-128), OF=1

Or ax,0    ; clear OF

Interrupt Flag

CLI - Disables the maskable hardware interrupts by clearing the Interrupt flag.

STI - Sets the Interrupt Flag to 1, which enables recognition of all hardware interrupts.

Data Transfer Instructions

MOV - Move Byte or Word

Usage:  MOV     dest,src

Copies byte or word from the source operand to the destination operand.

Operands

  - mem,accum

  - accum,mem

  - reg,immed

  - reg,reg

  - reg,mem

  - mem,reg

  - reg16,segreg

  - segreg,mem16

  - segreg,reg16

  - mem16,segreg

XCHG - Exchange

Usage:  XCHG    dest,src

Exchanges contents of source and destination.

Operands

reg,reg

mem,reg

reg,mem

accum,reg

reg,accum

Addition and subtraction

INC - Increment

Usage:  INC     dest

Adds one to destination.

  - Reg

  - Mem

DEC - Decrement

Usage:  DEC     dest

Subtracts one from the destination.

  - reg

  - mem

ADD - Arithmetic Addition

Usage:  ADD     dest,src

Adds "src" to "dest" and replacing the original contents of "dest".

Both operands are binary.

  - reg,reg

  - mem,reg

  - reg,mem

  - reg,immed

  - accum,immed

SUB - Subtract

Usage:  SUB     dest,src

The source is subtracted from the destination and the result is stored in the destination.

  - reg,reg

  - mem,reg

  - reg,mem

  - accum,immed

  - reg,immed

  - mem,immed

NEG - Two's Complement Negation

Usage:  NEG     dest

Subtracts the destination from 0 and saves the 2s complement of "dest" back into "dest".

Operands

reg

mem

JMP and LOOP instructions

JMP - Unconditional Jump

Usage:  JMP     target

Start:

…

…

Jmp Start

Unconditionally transfers control to "label".  Jumps by default are within -32768 to 32767 bytes from the instruction following the jump.  NEAR and SHORT jumps cause the IP to be updated while FAR jumps cause CS and IP to be updated.

A short jump is within -128 to +127 bytes from the current location. A near jump is within the same code segment, and a far jump is outside the current segment.

JMP ShortLabel

JMP NearLabel

JMP FarLabel

JMP RegWord

JMP mem16

JMP mem20

Conditional Jumps

| Jcc | Short | 0111cccc | Jump on Some Condition Code |
| --- | --- | --- | --- |

| CCCC | Name | Means | In short |
| --- | --- | --- | --- |
| 0000 | O | overflow | o=1 |
| 0001 | NO | Not overflow | o=0 |
| 0010 | C/B/NAE | Carry, below, not above nor equal | c=1 |
| 0011 | NC/AE/NB | Not carry, above or equal, not below | c=0 |
| 0100 | E/Z | Equal, zero | z=1 |
| 0101 | NE/NZ | Not equal, not zero | z=0 |
| 0110 | BE/NA | Below or equal, not above | c=1 or z=1 |
| 0111 | A/NBE | Above, not below nor equal | c=0 and z=0 |
| 1000 | S | Sign (negative) | s=1 |
| 1001 | NS | Not sign | s=0 |
| 1010 | P/PE | Parity, parity even | p=1 |
| 1011 | NP/PO | Not parity, parity odd | p=0 |
| 1100 | L/NGE | Less, not greater nor equal | s<>o |
| 1101 | GE/NL | Greater or egual, not less | s=o |
| 1110 | LE/NG | Less or equal, not greater | z=1 or s<>o |
| 1111 | G/NLE | Greater, not less nor equal | z=0 and s=o |

LOOP - Decrement CX and Loop if CX Not Zero

Usage:  LOOP    label

Decrements CX by 1 and transfers control to "label" if CX is not Zero.  The "label" operand must be within -128 or 127 bytes of the instruction following the loop instruction

Call and Ret instructions

CALL - Procedure Call

Usage:  CALL    destination

Pushes Instruction Pointer (and Code Segment for far calls) onto

stack and loads Instruction Pointer with the address of proc-name.

Code continues with execution at CS:IP.

Call NearLabel

Call FarLabel

Call RegWord

Call mem16

Call mem20

RET - Return From Procedure

Pops a return address from the stack. RETN (return near) pops of the stack into IP. In Real-address mode, RETF (return far) pops the stack first into IP, and then into CS. An optional 8-bit immediate operand tells the CPU to add a value to SP after popping the return address.

Operands

retn

retn immed

retf

retf immed

Multiplication and Division Instructions

MUL - Unsigned Multiply

Usage:  MUL     src

Unsigned multiply of the accumulator by the source.  If "src" is

a byte value, then AL is used as the other multiplicand and the

result is placed in AX.  If "src" is a word value, then AX is

multiplied by "src" and DX:AX receives the result.

Operands

  - reg8

  - reg16

  - mem8

  - mem16

| Multiplicand | Multiplier | Product |
| --- | --- | --- |
| AL | r/m8 | AX |
| AX | r/m16 | DX:AX |

DIV - Divide

Usage:  DIV     src

Unsigned binary division of accumulator by source.  If the source divisor is a byte value then AX is divided by "src" and the quotient is placed in AL and the remainder in AH.  If source operand is a word value, then DX:AX is divided by "src" and the quotient is stored in AX and the remainder in DX.

  - reg

  - mem

| Dividend | Divisor | Quotient | Remainder |
| --- | --- | --- | --- |
| AX | r/m8 | AL | AH |
| DX:AX | r/m16 | AX | DX |

INT - Interrupt

Usage:  INT     num

Initiates a software interrupt by pushing the flags, clearing the Trap and Interrupt Flags, pushing CS followed by IP and loading CS:IP with the value found in the interrupt vector table.  Execution then begins at the location addressed by the new CS:IP

IRET - Interrupt Return

Usage:  IRET

Returns control to point of interruption by popping IP, CS and then the Flags from the stack and continues execution at this location.  CPU exception interrupts will return to the instruction that cause the exception because the CS:IP placed on the stack during the interrupt is the address of the offending instruction.

LAHF - Load Register AH From Flags

Usage:  LAHF

Copies bits 0-7 of the flags register into AH.  This includes flags

AF, CF, PF, SF and ZF other bits are undefined.

AH := SF ZF xx AF xx PF xx CF

NOP - No Operation

Usage:  NOP

This is a do nothing instruction. It may be used inside a timing loop.

HLT - Halt CPU

Usage:   HLT

Stops the CPU until hardware interrupt occurs (The Interrupt flag must be set)

IN - Input Byte or Word from Port

Usage:  IN      accum,port

A byte or word is read from "port" and placed in AL or AX respectively.  If the port number is in the range of 0-255 it can be specified as an immediate, otherwise the port number must be specified in DX.

accum,immed8

accum,DX

OUT - Output Data to Port

Usage:  OUT     port,accum

Transfers byte in AL or word in AX to the specified hardware port address.  If the port number is in the range of 0-255 it can be specified as an immediate.  If greater than 255 then the  port number must be specified in DX.

Operands

immed8,accum

DX,accum

POP - Pop Word off Stack

Usage:  POP     dest

Transfers word at the current stack top (SS:SP) to the destination then increments SP by two to point to the new stack top.

Operands

reg16

mem16

seg

POPF - Pop Flags off Stack

Usage:  POPF

Pops word from stack into the Flags Register and then increments SP by 2.

PUSH - Push Word onto Stack

Usage:  PUSH    src

Decrements SP by the size of the operand and transfers one word from source to the stack top (SS:SP).

Operands

reg16

mem16

seg

PUSHF - Push Flags onto Stack

Usage:  PUSHF

Transfers the Flags Register onto the stack.  PUSHF saves a 16 bit

value.

…..

…