EGN 3420                         EXAM I                 Name________________________

Fa 96

SHOW ALL WORK!

Problem 1 (25 pts)

Consider the function  f(x) = ex + x2.   A third order truncated Taylor Series of f(x) expanded about the point x0  = a  and evaluated at the point x=2a is given below.

f3(2a) = ea (0 + 1a + 2a2 + 3a3 ) + a2

- Find  0 ,   1 ,    2 ,     3 ,  and    .

- For a = 0.1, find the truncation error  ET = f(0.2) - f3(0.2) to 6 places after the decimal point.

Work Area

………………………………………………………………………………………………

Ans.	0  = _____,   1  = _____,    2  = _____,     3  = _____,   = _____

ET = __________

EGN 3420                         EXAM I                 Name________________________

Fa 96

SHOW ALL WORK!

Problem 2 (30 pts)

The function f(x) = x4 - 5x2 + 4 has a root located between 0 and 1.5.  Fill in the tables below for the first three iterations of the Bisection Method and the False Position method.  Express all answers to four digits after the decimal point.

| Iteration | xl | xu | xr | f(xl) | f(xr) | eA |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.0000 | 1.5000 |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |

Bisection Method

| Iteration | xl | xu | xr | f(xl) | f(xu) | f(xr) | eA |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.0000 | 1.5000 |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |

False Position Method

EGN 3420                         EXAM I                 Name________________________

Fa 96

SHOW ALL WORK!

Problem 3 (20 pts)

Use the one point iteration method to locate the smaller root of the quadratic

f(x) = x2 - 10x + 1.  Choose g(x) = (x2 +1)/10.

Stop when the magnitude of the true error, ET falls below  0.00001

| i | xi | xi+1 = g(xi) | ET = True root - xi |
| --- | --- | --- | --- |
| 0 | 5.00000 |  |  |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |
| 6 |  |  |  |
| 7 |  |  |  |
| 8 |  |  |  |

EGN 3420                         EXAM I                 Name________________________

Fa 96

SHOW ALL WORK!

Problem 4 (25 pts)

Find the matrix inverse  A-1  to solve the system of equations  Ax = b  given below.

x	+	y	+	z	=	6

x	-	2y	+	z	=	0

4x	+	y	-	2z	=	0

Work Area

………………………………………………………………………………………………

Ans.        A-1 =                         ,     x = _____,   y = _____,   z = _____