CDA4527 (Fall 2007)

Homework 3 Solution

1.  Chapter 4, problem 8. ( 15  points)

a) Prefix Match 			Link Interface

11100000 				0

11100001 00000000 			1

11100001 				2

otherwise 				3

b) Prefix match for first address is 4th entry: link interface       3

Prefix match for second address is 2nd entry: link interface  1

Prefix match for first address is 3rd entry: link interface       2

2.  Chapter 4, problem 10 ( 15 points)

Destination Address Range 				Link Interface

10000000 ---  10111111       	(64 addresses) 			0

11000000 ---  11011111       	(32 addresses) 			1

11100000 --- 11111111	 (32 addresses) 		2

00000000 --- 01111111	 (128 addresses) 		3

3.  Chapter 4, problem 11 ( 15  points)

223.1.17.0/25

223.1.17.128/26

223.1.17.192/26

4.  Chapter 4, problem 22  (  20 points)

v

6.  (10 points)

The three major routing protocols used in the Internet are:  RIP, OSPF, and BGP.

RIP: Routing Information Protocol

OSPF: Open Shortest Path First protocol

BGP:   Border Gateway Protocol

OSPF is mainly based on link state algorithm, RIP and BGP are mainly based on distance vector algorithm.

5.  (25 points)

Round   1                                           Round 2                                                  Round 3

Node A Table

A      B      C      D              	      A      B      C      D    		      A      B      C      D

A    0      1       7      00 		A    0      1       6      4			A    0      1       6      4

B    00    00     00    00		B    1       0      5       3			B    1       0      5       3

C    00    00     00    00		C    7       5      0       2			C    6       5      0       2

Node B Table

A      B      C      D		      A      B      C      D			      A      B      C      D

A    00    00    00     00		A    0       1       7     00			A    0      1       6      4

B    1       0      5       3		B    1       0      5       3			B    1       0      5       3

C    00    00     00    00		C    7       5      0       2			C    6       5      0       2

D    00    00     00    00		D    00     3       2      0			D    4       3       2      0

Node C Table

A      B      C      D		     A      B      C      D			     A      B      C      D

A    00    00    00     00		A    0       1       7     00			A    0	   1      6       4

B    00    00    00     00		B    1       0      5       3			B    1       0      5       3

C    7       5      0       2		C    6       5      0       2			C    6       5      0       2

D    00    00     00    00		D    00     3       2      0			D    4       3       2      0

Node D Table

A      B      C      D		      A      B      C      D			      A      B      C      D

B    00    00     00    00		B    1       0      5       3			B    1       0      5       3

C    00    00     00    00		C    7       5      0       2			C    6       5      0       2

D    00     3       2      0		D    4       3       2      0			D    4       3       2      0

“00” represents “infinity”.  Node B only broadcasts its distance vector once; the other three nodes broadcast their distance vectors twice.