REVIEW QUESTIONS FOR MIDTERM  TEST.

An object is photographed after a rotation  about Y axis followed by a rotation about Z axis. Supposing it was first rotated by Z axis and then rotated by the Y axis by same amounts as before, and then photographed. Would the two images be different? Take a corner of the room as origin of world coordinate system.

Both the  mean filter (averaging filter) and the Gaussian filter  are used out for smoothing a noisy image. Which one would you prefer and under what circumstances?

The matrix below shows the gray values in an image segment. The value at one of the pixel is underlined. Using Prewit’s filter  find the directional derivative in the direction of the gradient for the  marked pixel.

30  30  30  30  30

- 30  30  30  70

- 30  30  70  70

- 30  70  70  70

- 70  70  70  70

- 70  70  70  70

What is the need of two thresholds in Canny Edge detection scheme?

Discuss the utility of Laplacian of  gaussian operator for edge detection as compared to sobel operator.