EEL 4781 - Homework 1

Due October 9, 2008

Problem 1:

You have a car which travels at an average speed of 65 miles per hour. You carry your harddrive with 1TB of data. For what range of distances your car has a higher data rate than an OC-3 line with 155Mbps?

Problem 2:

Which layer (or layers) handle(s):

- dividing the transmitted bit stream into frames

- determining the route to use

- error detection and correction

- flow control

Problem 3:

Frames are the units exchanged at the data link layer. Packets are the units exchanged at the network layer. Are:

- packets encapsulated in frames or

- frames encapsulated in packets?

Explain.

Problem 4:

Discuss the similarities and the differences between the TCP/IP model and the ISO model.

Problem 5:

Television channels are 6MHz wide. How many bits/sec can be sent if four level digital signals are used? Assume a noiseless channel.

Problem 6:

Why is the PCM sampling time set to 125 microseconds?

Problem 7:

Let us assume a CDMA system in which the chip sequence of the stations are:

A: -1 -1 -1 +1 +1 -1 +1 +1

B: -1 -1 +1 -1 +1 +1 +1 -1

C: -1 +1 -1 -1 -1 -1 +1 -1

D: -1 +1 -1 +1 +1 +1 -1 -1

A CDMA receiver gets the following chips:

-1 +1 -3 +1 -1 -3 +1 +1

Which stations transmitted, and what bits they have each transmitted?

Problem 8:

Explain the differences between CDMA, TDMA and FDMA.

Which ones can be combined? What would be the advantages and disadvantages of such combinations?

Problem 9:

An upper layer packet is split into 10 frames, each of them having an 80 percent chance of arriving undamaged (eg. a very noisy WiFi environment). Assume no error control at the data link layer.

- In average, how many times must the packet be resent at the higher level until it gets through?

- What about if the chance of a frame to arrive undamaged is 99.999%?

Note: you can either do probabilistic calculations, or you can write a small simulation program.

Problem 10:

The following encoding is used in a data link protocol:

A:    01000111

B:    11100011

C:    10101111

FLAG: 01111110

ESC:	 11100000

Show the bit sequence transmitted for the frame “A B C ESC FLAG” (this is the binary code, the ESC and FLAG should appear as such).

What is the output if the framing method is:

- Character count

- Flag bytes with byte shifting

- Starting and ending flag bytes with bit stuffing.

Problem 11:

- Find the CRC of 01010001 with the generator polynomial x3+1 (1001).

- Show the actual bit stream transmitted.

- Assume that the third bit from left is flipped in the transmission due to noise. Show how the error is detected at the receiving side.

Problem 12:

Trace the operation of a sliding window protocol with a window size 4 and selective retransmission for the transmission of 10 packets P1,P2… P10. Assume that every 3rd packet is damaged during transmission.