EEL 5708 – Midterm examination

Date: October 24, 2003

Name:

Student Code:

Instructions:

- This exam is open book and open notes. Allotted time is 90 minutes.

- Explicitly state all your assumptions.

- Don’t just give answers, always try to write down the way you were thinking. Even if you can not find a solution, write down the paths you have tried.

Problem 1 (20 points)

Give assembly language examples for RAW, WAR and WAW hazards. Show the hazards with arrows.

Problem 2 (20 points)

Concisely explain why speculation gives better performance than Tomasulo register renaming.

Problem 3 (20 points)

Explain the problem of precise interrupts. Give an example assembly program, where a naïve implementation of Tomasulo’s algorithm would not give precise interrupts. Explain how the speculative solution presented in class would solve this problem.

Problem 4 (20 points)

Consider the following assembly program written for a RISC style 32 bit

machine:

ld  r20, 16000     # load immediate

ld  r19, 30000

ld  r18, 60000

start:

add r3, r20, r19

ld  r1, 0(r3)      # load into r1 from memory

add r4, r20, r18

ld  r2, 0(r4)

mult r2, r1, r2

st  0(r4), r2      # store r2 into memory

ld  r2, 16

sub r20, r20, r2

bnez r20, start

# jump to start if r20 not equal with zero

- Write a C, C++ or Java program which corresponds to this assembly program

- Identify the RAW, WAR and WAW conflicts in the assembly program.

- Assume that the program is run on a machine which has a 5 stage pipeline. Rewrite the assembly program by inserting "nop" operations where there will be stalls.

- Unroll the loop in the program 4 times in software (assembly). Transform the program by register renaming such that the number of stalls is minimized.

Problem 5 (20 points)

Suppose that a processor with a load/store architecture executes at a clock rate of 2GHz, with the ideal CPI of  1.5. The typical applications run on this processor contain a mix of 60% arithmetic and logic instructions, 20% load and store instructions and 20% control instructions.

The processor accesses the memory through a separate data and instruction cache. An average 2% of the instructions produce an instruction miss, with a penalty of 25 clock cycles. An average of 12% of the data accesses are cache misses. The penalty of a data cache miss is 60 clock cycles. Cache hits do not produce any penalty.

- What is the real CPI of the architecture?

- What is the average memory access time (AMAT) of the architecture?